# PRACTICAL – 2

## AIM

To study and understand the different types of network cables used in computer networking.

## Different Types of Network Cables

Network cables are physical transmission media used to connect computers, networking devices, and other communication equipment. The major types of cables used in networking are:

  - Unshielded Twisted Pair (UTP) Cable

  - Shielded Twisted Pair (STP) Cable

  - Shielded and Screened Twisted Pair (SSTP) Cable

  - Coaxial Cable

  - Fibre Optic Cable

## UN-SHIELDED TWISTED PAIR (UTP) CABLE

UTP cable consists of pairs of insulated copper wires twisted together. It is one of the most commonly used cables in Ethernet networks.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 3 | 10 Mbps | Advantages: Low cost, easy to install, small diameter. Disadvantages: More susceptible to electromagnetic interference and noise. | 10Base-T Ethernet |
| Category 5 | Up to 100 Mbps | Provides better performance than older categories and is easy to install. | Fast Ethernet |
| Category 5e | Up to 1 Gbps | Improved performance and reduced interference compared with Cat 5. | Fast Ethernet, Gigabit Ethernet |
| Category 6 | Up to 10 Gbps | High speed and improved protection against interference. | Gigabit Ethernet, 10G Ethernet |
| Category 6a | Up to 10 Gbps | Supports high-speed data transmission over longer distances with reduced interference. | 10G Ethernet, data centres |

### Advantages of UTP

  - Cheaper in cost.

  - Easy to install and maintain.

  - Flexible and lightweight.

  - Smaller overall diameter.

  - Commonly available.

### Disadvantages of UTP

  - More prone to electromagnetic interference (EMI).

  - More susceptible to external noise than shielded cables.

  - Performance can be affected in environments with high electrical interference.

## SHIELDED TWISTED PAIR (STP) CABLE

STP cable is similar to UTP but contains additional shielding around the twisted wire pairs. The shielding helps reduce electromagnetic interference and improves signal quality.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 6 | Up to 10 Gbps | Advantages: Shielded, high speed, less susceptible to noise and interference. Disadvantages: More expensive and requires greater installation effort. | Gigabit Ethernet, 10G Ethernet |
| Category 6a | Up to 10 Gbps | Provides high bandwidth and improved protection from interference. | 10G Ethernet, data centres |

### Advantages of STP

  - Provides shielding against electromagnetic interference.

  - Less susceptible to noise.

  - Supports high-speed data transmission.

  - Suitable for environments with electrical interference.

### Disadvantages of STP

  - More expensive than UTP.

  - Installation is comparatively difficult.

  - Requires proper grounding for effective shielding.

  - Slightly thicker and less flexible than UTP.

## SHIELDED AND SCREENED TWISTED PAIR (SSTP) CABLE

SSTP cable provides additional shielding around the individual twisted pairs and/or the entire cable. It offers better protection from electromagnetic interference than ordinary UTP cables.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 7 | Up to 10 Gbps | Advantages: Excellent shielding, high bandwidth, low interference. Disadvantages: Expensive and requires careful installation. | Gigabit Ethernet, 10G Ethernet |

### Advantages of SSTP

  - Excellent protection from electromagnetic interference.

  - Supports high-speed data transmission.

  - Suitable for high-performance networking environments.

  - Provides better signal quality.

### Disadvantages of SSTP

  - Higher cost.

  - More difficult to install.

  - Requires proper grounding and compatible connectors.

## COAXIAL CABLE

Coaxial cable consists of a central copper conductor surrounded by insulation, a metallic shield, and an outer protective covering. It is commonly used for television and broadband communication.

### Types of Coaxial Cable

| Type | Typical Application |
| --- | --- |
| RG-59 | Television and video connections |
| RG-6 | Cable television and high-speed Internet connections |
| RG-11 | Long-distance cable and broadband applications |

### Data Transmission

Coaxial cables can support data rates ranging from approximately 10 Mbps to 100 Mbps or higher, depending on the cable type and network technology.

### Advantages

  - High bandwidth.

  - Good resistance to electromagnetic interference.

  - Low signal loss.

  - Reliable transmission.

  - Versatile and widely used.

### Disadvantages

  - Bulky compared with twisted-pair cables.

  - More difficult to install.

  - Limited transmission distance without signal amplification.

  - Generally more expensive than basic UTP cable.

### Applications

  - Cable television networks.

  - Broadband Internet connections.

  - CCTV and video transmission.

  - Older Ethernet networks.

## FIBRE OPTIC CABLE

Fibre optic cable transmits data in the form of light pulses through thin strands of glass or plastic fibre. It provides very high speed, bandwidth, and transmission distance.

### Types of Fibre Optic Cable

- Single-Mode Fibre

  - Uses a very small core.

  - Suitable for long-distance communication.

  - Provides high bandwidth and low signal loss.

- Multi-Mode Fibre

  - Has a larger core than single-mode fibre.

  - Suitable for shorter-distance communication.

  - Commonly used in LANs and data centres.

| Type | Data Transmission | Main Application |
| --- | --- | --- |
| Single-Mode Fibre | Very high speed, commonly used for long-distance links | Telecommunications, WANs, long-distance networks |
| Multi-Mode Fibre | Up to 100 Gbps and beyond depending on the standard | LANs, data centres, campus networks |

### Advantages

- Very high data transmission speed.

- Very high bandwidth.

- Suitable for long-distance communication.

- Highly secure and difficult to tap.

- Immune to electromagnetic interference.

- Low signal attenuation.

- Lightweight compared with copper cables.

### Disadvantages

- Higher installation cost.

- Requires skilled installers.

- Fibre is more delicate than copper cable.

- Special equipment is required for splicing and termination.

### Applications

- Internet backbone networks.

- Telecommunications.

- Data centres.

- High-speed LANs.

- Long-distance communication systems.

## COMPARISON OF NETWORK CABLES

| Cable Type | Transmission Medium | Speed | Interference Protection | Common Applications |
| --- | --- | --- | --- | --- |
| UTP | Copper | Up to 10 Gbps depending on category | Low to moderate | LAN, Ethernet |
| STP | Shielded copper | Up to 10 Gbps | High | Gigabit Ethernet, industrial networks |
| SSTP | Shielded copper | Up to 10 Gbps | Very high | 10G Ethernet, data centres |
| Coaxial | Copper | 10–100 Mbps or higher depending on technology | Good | TV, broadband, CCTV |
| Fibre Optic | Glass/plastic fibre | Up to 100 Gbps or higher | Immune to EMI | Data centres, Internet backbone, WAN |

## CONCLUSION

Thus, the different types of network cables such as UTP, STP, SSTP, coaxial cable, and fibre optic cable were studied. Each cable has different characteristics, transmission speeds,

advantages, disadvantages, and applications. UTP is economical and widely used for LAN connections, shielded cables are suitable for environments with interference, coaxial cables are commonly used for broadband and television networks, and fibre optic cables are preferred for high-speed and long-distance communication.